CRC

CRC Tx:

module crc_tx(input clk,

input enable,

input rst,

input [6:0] data_in,

output [9:0]data_out,

output [2:0] crc_out);

reg [9:0]tmp_data_out;

reg [2:0]tmp_crc_out;

function [2:0] compute_crc;  // returns crc out value

input [6:0]data;

reg [3:0]crc;

integer i;

begin

crc = 4'b0000;

for (i=7;i>=0;i=i-1)begin

crc = crc<<1;

if(crc[3] ^ data[i])

crc= crc^4'b1001;   //x3+1 polynomial

end

compute_crc=crc[2:0];

end

endfunction

always @ (posedge clk)begin

if(rst)begin

tmp_data_out <= 10'd0;

tmp_crc_out <= 3'd0;

end

else if (enable)begin

tmp_crc_out <=compute_crc(data_in);

tmp_data_out <={data_in,compute_crc(data_in)};

end

end

assign data_out=tmp_data_out;

assign crc_out=tmp_crc_out;

endmodule

CRC Rx:

module crc_rx(input clk,

input enable,

input rst,

input [6:0] data_in,

input [2:0]crc_received,

output reg [2:0]crc_out,

output  data_valid );

function [2:0] compute_crc(input [6:0] data);

reg [3:0]crc;

integer i;

begin

crc = 4'b0000;

for (i=6;i>=0;i=i-1)begin

crc = crc<<1;

if(crc[3] ^ data[i])

crc= crc^4'b1001;   //x3+1 polynomial

end

compute_crc = crc[2:0];

end

endfunction

always @ (posedge clk)begin

if(rst)begin

crc_out<=3'b000;

//data_valid<=1'b0;

end

else if (enable)begin

crc_out<=compute_crc(data_in);

end

end

assign data_valid=(crc_out==crc_received)?1:0;

endmodule

CRC Tx-TB:

module tb(  );

reg clk;

reg enable;

reg rst;

reg [6:0] data_in;

wire [9:0] data_out;

wire [2:0] crc_out;

crc_tx dut(.clk(clk),

.enable(enable),

.rst(rst),

.data_in(data_in),

.data_out(data_out),

.crc_out(crc_out));

always #5 clk=~clk;

initial begin

$monitor(" clk:%b ; en:%b ; rst:%b ;data in:%b ; crc out:%b; data out:%b",

clk,enable,rst,data_in,crc_out,data_out);

clk=0;enable=0;rst=1;data_in=7'd0;

#10 rst =0;

#10 data_in=7'b1010000;enable=1;

#10 enable =0;

#10 data_in=7'b1110011;enable=1;

#10 enable =0;

#10 data_in=7'b1010101;enable=1;

#10 enable =0;

#20 $finish;

end

endmodule

CRC Rx-TB:

module tb_rx( );

reg clk,enable,rst;

reg [6:0] data_in;

reg [2:0] crc_received;

wire [2:0] crc_out;

wire data_valid;

crc_rx dut (.clk(clk),

.rst(rst),

.enable(enable),

.data_in(data_in),

.crc_received(crc_received),

.crc_out(crc_out),

.data_valid(data_valid));

always #5 clk=~clk;

initial begin

$monitor(" clk: %b, rst:%b, en:%b, data:%b, crc_in:%b, crc_out:%b, valid:%b",

clk,rst,enable,data_in,crc_received, crc_out, data_valid);

clk=0; rst=1; enable=0; data_in=7'd0; crc_received=3'd0;

#10 rst=0;

#10 data_in=7'b1010000; crc_received=3'b011; enable=1;

#10 enable=0;

#10 data_in=7'b1110011; crc_received=3'b100; enable=1;

#10 enable=0;

#10 data_in=7'b1010101; crc_received= 3'b111; enable=1;

#10 enable=0;

$display("COMP CRC%d", dut.compute_crc(data_in));

#10 $finish;

end

endmodule

CRC TOP-TB:

module top_tb( );

reg clk,rst,enable;   // common for both tx and rx

// tx

reg [6:0]tx_data_in;

wire [2:0] tx_crc_out;

wire [9:0] tx_data_out;   //[9:3] data [2:0] crc

// rx

wire [2:0] rx_crc_out;

wire data_valid;

crc_tx transmitter(.clk(clk),

.enable(enable),

.rst(rst),

.data_in(tx_data_in),

.data_out(tx_data_out),

.crc_out(tx_crc_out));

crc_rx receiver(.clk(clk),

.enable(enable),

.rst(rst),

.data_in(tx_data_out[9:3]),

.crc_received(tx_data_out[2:0]),

.crc_out(rx_crc_out),

.data_valid(data_valid));

always #5 clk=~clk;

initial begin

$monitor("clk=%b;en=%b;rst=%b;tx data =%b;rx data=%b;tx crc=%b;rx crc=%b,valid=%b",

clk,enable,rst,tx_data_in,tx_data_out[9:3],tx_crc_out,rx_crc_out,data_valid);

clk=0 ; enable=0 ; rst =1 ;tx_data_in =7'd0;

#10 rst =0;enable =1;tx_data_in=7'b1010000;

#20;

tx_data_in = 7'b1010110;

#30 $finish;

end

endmodule